Static Tunnel Test Case

January 29 , 2024​​

15:17

DUT1:

evpn nve source-address 1.1.1.1

!

!

vlan 1-4094

!

nvi 10

vxlan-id 5010

!

nvi 20

vxlan-id 5020

!

Interface Ethernet1/0/1

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet1/0/2

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet1/0/33

switchport access vlan 1 2

!

interface Vlan1 2

ip address 1 2 .1.1.1 255.255.255.0

!

interface Loopback1

IP address 1.1.1.1 255.255.255.255

!

router ospf 1

ospf router-id 1.1.1.1

passive-interface Loopback1

network 1.1.1.1 0.0.0.0 area 0

network 1 2 .1.1.1 0.0.0.0 area 0

!

interface Nve301

nve mode vxlan

source 1.1.1.1

destination 2.2.2.2

join nvi 10

join nvi 20

DUT2:

evpn nve source-address 2.2.2.2

!

!

vlan 1-4094

!

nvi 10

vxlan-id 5010

!

nvi 20

vxlan-id 5020

!

Interface Ethernet1/0/1

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet1/0/49

switchport access vlan 12

!

interface Vlan 12

IP address 12.1.1.2 255.255.255.0

!

interface Loopback1

IP address 2.2.2.2 255.255.255.255

!

router ospf 1

ospf router-id 2.2.2.2

passive-interface Loopback1

network 2.2.2.2 0.0.0.0 area 0

network 12 .1.1.2 0.0.0.0 area 0

!

interface Nve301

nve mode vxlan

source 2.2.2.2

destination 1.1.1.1

join nvi 10

join nvi 20

S5:

vlan 1-4094

!

!

Interface s5p15

switchport mode trunk

!

Interface s5p11

switchport mode trunk

interface Nve301

nve mode vxlan

source 1.1.1.1

destination 2.2.2.2

join nvi 1001

join nvi 1002

Dynamic tunnel test case

January 29 , 2024​​

15:32

Initial configuration

DUT1:

evpn nve source-address 1.1.1.1

!

!

vlan 1-4094

!

nvi 10

vxlan-id 5010

evpn

rd 10:1

route-target both 10:12

route-target both 10:13

enable

evpn-exit

!

nvi 20

vxlan-id 5020

evpn

rd 20:1

route-target both 20:12

route-target both 20:13

enable

evpn-exit

!

Interface Ethernet1/0/1

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet1/0/2

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet1/0/33

switchport access vlan 13

!

interface Vlan13

IP address 13.1.1.1 255.255.255.0

!

interface Loopback1

IP address 1.1.1.1 255.255.255.255

!

router ospf 1

ospf router-id 1.1.1.1

passive-interface Loopback1

network 1.1.1.1 0.0.0.0 area 0

network 13.1.1.1 0.0.0.0 area 0

!

router bgp 100

neighbor 3.3.3.3 remote-as 100

neighbor 3.3.3.3 update-source 1.1.1.1

address-family l2vpn evpn

neighbor 3.3.3.3 activate

exit-address-family

!

DUT3

evpn nve source-address 3.3.3.3

!

!

vlan 1-4094

!

nvi 10

vxlan-id 5010

evpn

10:3

route-target both 10:13

route-target both 10:23

enable

evpn-exit

!

nvi 20

vxlan-id 5020

evpn

rd 20:3

route-target both 20:13

route-target both 20:23

enable

evpn-exit

!

Interface Ethernet1/0/1

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet1/0/49

switchport access vlan 23

!

Interface Ethernet1/0/50

switchport access vlan 13

!

interface Vlan13

IP address 13.1.1.3 255.255.255.0

!

interface Vlan23

IP address 23.1.1.3 255.255.255.0

!

interface Loopback1

IP address 3.3.3.3 255.255.255.255

!

router ospf 1

passive-interface Loopback1

network 3.3.3.3 0.0.0.0 area 0

network 13.1.1.3 0.0.0.0 area 0

network 23.1.1.3 0.0.0.0 area 0

!

router bgp 100

neighbor ibgp peer-group

neighbor ibgp remote-as 100

neighbor ibgp update-source 3.3.3.3

neighbor 1.1.1.1 peer-group ibgp

neighbor 2.2.2.2 peer-group ibgp

address-family l2vpn evpn

neighbor 1.1.1.1 activate

neighbor 1.1.1.1 route-reflector-client

neighbor 2.2.2.2 activate

neighbor 2.2.2.2 route-reflector-client

exit-address-family

!

DUT2:

evpn nve source-address 2.2.2.2

!

!

vlan 1-4094

!

nvi 10

vxlan-id 5010

evpn

10:2

route-target both 10:12

route-target both 10:23

enable

evpn-exit

!

nvi 20

vxlan-id 5020

evpn

rd 20:2

route-target both 20:12

route-target both 20:23

enable

evpn-exit

!

port-group 1

!

Interface Ethernet1/0/1

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet1/0/49

switchport access vlan 23

!

interface Vlan23

IP address 23.1.1.2 255.255.255.0

!

interface Loopback1

IP address 2.2.2.2 255.255.255.255

!

router ospf 1

ospf router-id 2.2.2.2

passive-interface Loopback1

network 2.2.2.2 0.0.0.0 area 0

network 23.1.1.2 0.0.0.0 area 0

!

router bgp 100

neighbor 3.3.3.3 remote-as 100

neighbor 3.3.3.3 update-source 2.2.2.2

address-family l2vpn evpn

neighbor 3.3.3.3 activate

exit-address-family

!

S5:

vlan 1-4094

!

!

Interface s5p15

switchport mode trunk

!

Interface s5p11

switchport mode trunk

Centralized gateway test cases (dynamic / static)

February 18 , 2024​​

18:08

Dynamically configure centralized gateway

DUT1:

evpn nve source-address 1.1.1.1

!

!

vlan 1-4094

!

nvi 10

vxlan-id 5010

evpn

rd 10:1

route-target both 10:13

enable

evpn-exit

!

nvi 20

vxlan-id 5020

evpn

rd 20:1

route-target both 20:13

enable

evpn-exit

!

Port-group 1

!

Interface Ethernet s1p11

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet s1p15

switchport mode trunk

port-group 1 mode active

!

Interface Ethernet s1p13

switchport access vlan 13

!

Interface Port-Channel1

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

interface Vlan13

IP address 13.1.1.1 255.255.255.0

!

interface Loopback1

IP address 1.1.1.1 255.255.255.255

!

router ospf 1

ospf router-id 1.1.1.1

passive-interface Loopback1

network 1.1.1.1 0.0.0.0 area 0

network 13.1.1.1 0.0.0.0 area 0

!

router bgp 100

neighbor 3.3.3.3 remote-as 100

neighbor 3.3.3.3 update-source 1.1.1.1

address-family l2vpn evpn

neighbor 3.3.3.3 activate

exit-address-family

!

DUT3

evpn nvi-vlan-mapping-monitor disable

evpn nve source-address 3.3.3.3

!

!

vlan 1-4094

!

nvi 10

vxlan-id 5010

evpn

10:3

route-target both 10:13

route-target both 10:23

enable

evpn-exit

!

nvi 20

vxlan-id 5020

evpn

rd 20:3

route-target both 20:13

route-target both 20:23

enable

evpn-exit

!

Interface Ethernet1/0/1

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet1/0/49

switchport access vlan 23

!

Interface Ethernet1/0/50

switchport access vlan 13

!

interface Vlan13

IP address 13.1.1.3 255.255.255.0

!

interface Vlan23

IP address 23.1.1.3 255.255.255.0

!

interface Loopback1

IP address 3.3.3.3 255.255.255.255

!

interface Nvi-interface10

mac-address 00-00-5e-00-00-10

ipv6 address 2010::254/64

IP address 192.168.10.254 255.255.255.0

!

interface Nvi-interface20

mac-address 00-00-5e-00-00-20

ipv6 address 2020::254/64

IP address 192.168.20.254 255.255.255.0

!

router ospf 1

passive-interface Loopback1

network 3.3.3.3 0.0.0.0 area 0

network 13.1.1.3 0.0.0.0 area 0

network 23.1.1.3 0.0.0.0 area 0

!

router bgp 100

neighbor ibgp peer-group

neighbor ibgp remote-as 100

neighbor ibgp update-source 3.3.3.3

neighbor 1.1.1.1 peer-group ibgp

neighbor 2.2.2.2 peer-group ibgp

address-family l2vpn evpn

neighbor 1.1.1.1 activate

neighbor 1.1.1.1 route-reflector-client

neighbor 2.2.2.2 activate

neighbor 2.2.2.2 route-reflector-client

exit-address-family

!

DUT2:

evpn nve source-address 2.2.2.2

!

!

vlan 1-4094

!

nvi 10

vxlan-id 5010

evpn

10:2

route-target both 10:23

enable

evpn-exit

!

nvi 20

vxlan-id 5020

evpn

rd 20:2

route-target both 20:23

enable

evpn-exit

!

port-group 1

!

Interface Ethernet1/0/1

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet1/0/49

switchport access vlan 23

!

interface Vlan23

IP address 23.1.1.2 255.255.255.0

!

interface Loopback1

IP address 2.2.2.2 255.255.255.255

!

router ospf 1

ospf router-id 2.2.2.2

passive-interface Loopback1

network 2.2.2.2 0.0.0.0 area 0

network 23.1.1.2 0.0.0.0 area 0

!

router bgp 100

neighbor 3.3.3.3 remote-as 100

neighbor 3.3.3.3 update-source 2.2.2.2

address-family l2vpn evpn

neighbor 3.3.3.3 activate

exit-address-family

!

S5:

vlan 1-4094

!

!

Interface s5p15

switchport mode trunk

!

Interface s5p11

switchport mode trunk

ipv4 address vlan10 interface gateway address is 192.168.10.254 ,

vlan20 is 192.168.20.254 ;

the IPv6 address vlan10 interface is 2010::254 .

Vlan20 interface is 2020::254 ;

Tc1

Interface 1 : 192.168.10.10/24 00-00-11-01-00-01 vlan10

Interface 2 : 192.168.20.10/24 00-00-11-02-00-01 vlan20

Interface 3 : 2010::10/64 00-00-11-03-00-01 vlan10

Interface 4 : 2020::10/64 00-00-11-04-00-01 vlan20

Tc 2

Interface 1 : 192.168.10.20/24 00-00-22-01-00-01 vlan10

Interface 2 : 192.168.20.20/24 00-00-22-02-00-01 vlan20

Interface 3 : 2010::20/64 00-00-22-03-00-01 vlan10

Interface 4 : 2020::20/64 00-00-22-04-00-01 vlan20

Tc 3

Interface 1 : 192.168.10.30/24 00-00-33-01-00-01 vlan10

Interface 2 : 192.168.20.30/24 00-00-33-02-00-01 vlan20

Interface 3 : 2010::30/64 00-00-33-04-00-01 vlan10

Interface 4 : 2020::30/64 00-00-33-05-00-01 vlan20

Tc 4

Interface 1 : 192.168.10.40/24 00-00-44-01-00-01 vlan10

Interface 2 : 192.168.20.40/24 00-00-44-02-00-01 vlan20

Interface 3 : 2010::40/64 00-00-44-03-00-01 vlan10

Interface 4 : 2020::40/64 00-00-44-04-00-01 vlan20

Tc 5

Interface 1 : 192.168.10.50/24 00-00-55-01-00-01 vlan10

Interface 2 : 192.168.20.50/24 00-00-55-02-00-01 vlan20

Interface 3 : 2010::50/64 00-00-55-03-00-01 vlan10

Interface 4 : 2020::50/64 00-00-55-04-00-01 vlan20

Statically configure a centralized gateway

DUT1:

vlan 1-4094

!

nvi 10

vxlan-id 5010

!

nvi 20

vxlan-id 5020

!

Interface Ethernet1/0/1

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet1/0/2

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet1/0/33

switchport access vlan 13

!

interface Vlan13

IP address 13.1.1.1 255.255.255.0

!

interface Loopback1

IP address 1.1.1.1 255.255.255.255

!

router ospf 1

ospf router-id 1.1.1.1

passive-interface Loopback1

network 1.1.1.1 0.0.0.0 area 0

network 13.1.1.1 0.0.0.0 area 0

!

interface Nve 13

nve mode vxlan

source 1.1.1.1

destination 3.3.3.3

join nvi 10

join nvi 20

DUT3

vlan 1-4094

!

nvi 10

vxlan-id 5010

!

nvi 20

vxlan-id 5020

!

Interface Ethernet1/0/1

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet1/0/49

switchport access vlan 23

!

Interface Ethernet1/0/50

switchport access vlan 13

!

interface Vlan13

IP address 13.1.1.3 255.255.255.0

!

interface Vlan23

IP address 23.1.1.3 255.255.255.0

!

interface Loopback1

IP address 3.3.3.3 255.255.255.255

!

interface Nvi-interface10

mac-address 00-00-5e-00-00-10

ipv6 address 2010::254/64

IP address 192.168.10.254 255.255.255.0

!

interface Nvi-interface20

mac-address 00-00-5e-00-00-20

ipv6 address 2020::254/64

IP address 192.168.20.254 255.255.255.0

!

interface Nve13

nve mode vxlan

source 3.3.3.3

destination 1.1.1.1

join nvi 10

join nvi 20

!

interface Nve23

nve mode vxlan

source 3.3.3.3

destination 2.2.2.2

join nvi 10

join nvi 20

!

router ospf 1

passive-interface Loopback1

network 3.3.3.3 0.0.0.0 area 0

network 13.1.1.3 0.0.0.0 area 0

network 23.1.1.3 0.0.0.0 area 0

!

DUT2:

vlan 1-4094

!

nvi 10

vxlan-id 5010

!

nvi 20

vxlan-id 5020

!

Interface Ethernet1/0/1

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet1/0/49

switchport access vlan 23

!

interface Vlan23

IP address 23.1.1.2 255.255.255.0

!

interface Loopback1

IP address 2.2.2.2 255.255.255.255

!

router ospf 1

ospf router-id 2.2.2.2

passive-interface Loopback1

network 2.2.2.2 0.0.0.0 area 0

network 23.1.1.2 0.0.0.0 area 0

!

!

interface Nve 23

nve mode vxlan

source 2.2.2.2

destination 3.3.3.3

join nvi 10

join nvi 20

S5:

vlan 1-4094

!

!

Interface s5p15

switchport mode trunk

!

Interface s5p11

switchport mode trunk

Distributed Gateway Test Cases

March 25 , 2024​​

13:27

DUT1:

evpn nve source-address 1.1.1.1

evpn nve mac-address 00-00-00- 00-00-11

!

vlan 1-4094

!

ip vrf 1

rd 1 3:1

l3-vni 10001

route-target both 1:10001

!

ip vrf 2

rd 1 3:2

l3-vni 10002

route-target both 2:10002

!

vxlan remote mac-address-learning disable

vxlan remote arp-learning disable

vxlan remote nd-learning disable

!

nvi 10

vxlan-id 5010

evpn

rd 1:1 0

route-target both 1 0 : 5010

enable

evpn-exit

!

nvi 1 1

vxlan-id 501 1

evpn

rd 1:1 1

route-target both 1 1 : 5011

enable

evpn-exit

!

nvi 20

vxlan-id 5020

evpn

rd 1 :20

route-target both 2 0 : 5020

enable

evpn-exit

!

Port-group 1

!

Interface Ethernet s1p11

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 1 1 mode vlan svid 1 1

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet s1p15

switchport mode trunk

port-group 1 mode active

!

Interface Ethernet s1p13

switchport access vlan 13

!

Interface Port-Channel1

xconnect nvi 10 mode vlan svid 10

xconnect nvi 1 1 mode vlan svid 1 1

xconnect nvi 20 mode vlan svid 20

!

interface Vlan13

IP address 13.1.1.1 255.255.255.0

!

interface Loopback1

IP address 1.1.1.1 255.255.255.255

!

interface Nvi-interface10

ip vrf forwarding 1

mac-address 00-00-5e-00-00- 01

distributed-gateway enable

ipv6 address 2010::254/64

IP address 192.168.10.254 255.255.255.0

!

interface Nvi-interface1 1

ip vrf forwarding 1

mac-address 00-00-5e- 00-00-11

distributed-gateway enable

ipv6 address 201 1 ::254/64

IP address 192.168.1 1 .254 255.255.255.0

!

interface Nvi-interface20

ip vrf forwarding 2

mac-address 00-00-5e-00-00- 02

distributed-gateway enable

ipv6 address 20 2 0::254/64

IP address 192.168. 2 0.254 255.255.255.0

!

router ospf 1

ospf router-id 1.1.1.1

passive-interface Loopback1

network 1.1.1.1 0.0.0.0 area 0

network 13.1.1.1 0.0.0.0 area 0

!

router bgp 100

neighbor 3.3.3.3 remote-as 100

neighbor 3.3.3.3 update-source 1.1.1.1

address-family l2vpn evpn

neighbor 3.3.3.3 activate

exit-address-family

address-family ipv4 vrf 1

exit-address-family

address-family ipv4 vrf 2

exit-address-family

!

DUT3

evpn nve source-address 3.3.3.3

evpn nve mac-address 00-00-00- 00-00-33

!

vlan 1-4094

!

ip vrf 1

rd 3 3:1

l3-vni 10001

route-target both 1:10001

!

ip vrf 2

rd 3 3:2

l3-vni 10002

route-target both 2:10002

!

vxlan remote mac-address-learning disable

vxlan remote arp-learning disable

vxlan remote nd-learning disable

!

nvi 10

vxlan-id 5010

evpn

rd 3:10

route-target both 1 0 : 5010

enable

evpn-exit

!

!

nvi 1 1

vxlan-id 501 1

evpn

rd 3:11

route-target both 1 1 : 5011

enable

evpn-exit

!

nvi 20

vxlan-id 5020

evpn

rd 3:20

route-target both 2 0 : 5020

enable

evpn-exit

!

Interface Ethernet s3p11

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet s3p23

switchport access vlan 23

!

Interface Ethernet s3p13

switchport access vlan 13

!

interface Vlan13

IP address 13.1.1.3 255.255.255.0

!

interface Vlan23

IP address 23.1.1.3 255.255.255.0

!

interface Loopback1

IP address 3.3.3.3 255.255.255.255

!

interface Nvi-interface10

ip vrf forwarding 1

mac-address 00-00-5e-00-00- 01

distributed-gateway enable

ipv6 address 2010::254/64

IP address 192.168.10.254 255.255.255.0

!

!

interface Nvi-interface1 1

ip vrf forwarding 1

mac-address 00-00-5e- 00-00-11

distributed-gateway enable

ipv6 address 201 1 ::254/64

IP address 192.168.1 1 .254 255.255.255.0

!

interface Nvi-interface20

ip vrf forwarding 2

mac-address 00-00-5e-00-00- 02

distributed-gateway enable

ipv6 address 20 2 0::254/64

IP address 192.168. 2 0.254 255.255.255.0

!

router ospf 1

passive-interface Loopback1

network 3.3.3.3 0.0.0.0 area 0

network 13.1.1.3 0.0.0.0 area 0

network 23.1.1.3 0.0.0.0 area 0

!

router bgp 100

neighbor ibgp peer-group

neighbor ibgp remote-as 100

neighbor ibgp update-source 3.3.3.3

neighbor 1.1.1.1 peer-group ibgp

neighbor 2.2.2.2 peer-group ibgp

address-family l2vpn evpn

neighbor 1.1.1.1 activate

neighbor 1.1.1.1 route-reflector-client

neighbor 2.2.2.2 activate

neighbor 2.2.2.2 route-reflector-client

exit-address-family

address-family ipv4 vrf 1

exit-address-family

address-family ipv4 vrf 2

exit-address-family

!

DUT2:

evpn nve source-address 2.2.2.2

evpn nve mac-address 00-00-00- 00-00-22

!

vlan 1-4094

!

ip vrf 1

rd 2 3:1

l3-vni 10001

route-target both 1:10001

!

ip vrf 2

rd 2 3:2

l3-vni 10002

route-target both 2:10002

!

vxlan remote mac-address-learning disable

vxlan remote arp-learning disable

vxlan remote nd-learning disable

!

nvi 10

vxlan-id 5010

evpn

rd 2:10

route-target both 1 0 : 5010

enable

evpn-exit

!

!

nvi 1 1

vxlan-id 501 1

evpn

rd 2:11

route-target both 1 1 : 5011

enable

evpn-exit

!

nvi 20

vxlan-id 5020

evpn

rd 2:20

route-target both 2 0 : 5020

enable

evpn-exit

!

Interface Ethernet s2p11

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet s2p23

switchport access vlan 23

!

interface Vlan23

IP address 23.1.1.2 255.255.255.0

!

interface Loopback1

IP address 2.2.2.2 255.255.255.255

!

interface Nvi-interface10

ip vrf forwarding 1

mac-address 00-00-5e-00-00- 01

distributed-gateway enable

ipv6 address 2010::254/64

IP address 192.168.10.254 255.255.255.0

!

interface Nvi-interface1 1

ip vrf forwarding 1

mac-address 00-00-5e- 00-00-11

distributed-gateway enable

ipv6 address 201 1 ::254/64

IP address 192.168.1 1 .254 255.255.255.0

!

interface Nvi-interface20

ip vrf forwarding 2

mac-address 00-00-5e-00-00- 02

distributed-gateway enable

ipv6 address 20 2 0::254/64

IP address 192.168. 2 0.254 255.255.255.0

!

router ospf 1

ospf router-id 2.2.2.2

passive-interface Loopback1

network 2.2.2.2 0.0.0.0 area 0

network 23.1.1.2 0.0.0.0 area 0

!

router bgp 100

neighbor 3.3.3.3 remote-as 100

neighbor 3.3.3.3 update-source 2.2.2.2

address-family l2vpn evpn

neighbor 3.3.3.3 activate

exit-address-family

address-family ipv4 vrf 1

exit-address-family

address-family ipv4 vrf 2

exit-address-family

!

S5:

vlan 1-4094

!

Interface s5p15

switchport mode trunk

!

Interface s5p11

switchport mode trunk

ipv4 address vlan10 interface gateway address is 192.168.10.254 ,

vlan20 is 192.168.20.254 ;

the IPv6 address vlan10 interface is 2010::254 .

Vlan20 interface is 2020::254 ;

Tc1

Interface 1 : 192.168.10.10/24 00-00-11-01-00-01 vlan10

Interface 2 : 192.168.11.10/24 00-00-11-02-00-01 vlan11

Interface 3 : 192.168.20.10/24 00-00-11-03-00-01 vlan20

Interface 4 : 2010::10/64 00-00-11-04-00-01 vlan10

Interface 5 : 2011::10/64 00-00-11-05-00-01 vlan11

Interface 6 : 2020::10/64 00-00-11-06-00-01 vlan20

Tc 2

Interface 1 : 192.168.10.20/24 00-00-22-01-00-01 vlan10

Interface 2 : 192.168.11.20/24 00-00-22-02-00-01 vlan11

Interface 3 : 192.168.20.20/24 00-00-22-03-00-11 vlan20

Interface 4 : 2010::20/64 00-00-22-04-00-01 vlan10

Interface 5 : 2011::20/64 00-00-22-05-00-01 vlan11

Interface 6 : 2020::20/64 00-00-22-06-00-01 vlan20

Tc 3

Interface 1 : 192.168.10.30/24 00-00-33-01-00-01 vlan10

Interface 2 : 192.168.11.30/24 00-00-33-02-00-01 vlan11

Interface 3 : 192.168.20.30/24 00-00-33-03-00-11 vlan20

Interface 4 : 2010::30/64 00-00-33-05-00-01 vlan10

Interface 5 : 2011::30/64 00-00-33-06-00-01 vlan11

Interface 6 : 2020::20/64 00-00-33-07-00-01 vlan20

Tc 5

Interface 1 : 192.168.10.50/24 00-00-55-01-00-01 vlan10

Interface 2 : 192.168.11.50/24 00-00-55-02-00-01 vlan11

Interface 3 : 192.168.20.50/24 00-00-55-03-00-01 vlan20

Interface 4 : 2010::50/64 00-00-55-04-00-01 vlan10

Interface 5 : 2011::50/64 00-00-55-05-00-01 vlan11

Interface 6 : 2020::50/64 00-00-55-06-00-01 vlan20

Centralized Active-Active Gateway Scenario Test Case VEG

January 26 , 2024​​

13:19

DUT1

evpn nve source-address 1.1.1.1

evpn nve mac-address 00-00-00- 00-00-11

!

vlan 1-4094

!

vxlan remote mac-address-learning disable

!

nvi 10

vxlan-id 5010

evpn

rd 10: 1

route-target both 1 0 : 5010

enable

evpn-exit

!

nvi 20

vxlan-id 5020

evpn

rd 20: 1

route-target both 2 0 : 5020

enable

evpn-exit

!

Interface Ethernet s1p11

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet s1p15

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet s1p14

switchport access vlan 14

!

Interface Ethernet s1p13

switchport access vlan 13

!

interface Vlan13

IP address 13.1.1. 1 255.255.255.0

!

interface Vlan1 4

ip address 1 4 .1.1. 1 255.255.255.0

!

interface Loopback1

IP address 1.1.1.1 255.255.255.255

!

router ospf 1

passive-interface Loopback1

network 1.1.1.1 0.0.0.0 area 0

network 13.1.1. 1 0.0.0.0 area 0

network 1 4 .1.1. 1 0.0.0.0 area 0

!

router bgp 100

neighbor 3.3.3.3 remote-as 100

neighbor 3.3.3.3 update-source 1.1.1.1

neighbor 4.4.4.4 remote-as 100

neighbor 4.4.4.4 update-source 1.1.1.1

address-family l2vpn evpn

neighbor 3.3.3.3 activate

neighbor 4.4.4.4 activate

exit-address-family

!

DUT3

evpn nve source-address 3 4 .3 4 .3 4 .3 4

evpn nve mac-address 00-00-00- 00-00-34

!

vlan 1-4094

!

ip vrf 1

rd 3 3:1

l3-vni 10001

route-target both 1:10001

!

ip vrf 2

rd 3 3:2

l3-vni 10002

route-target both 2:10002

!

nvi 10

vxlan-id 5010

evpn

rd 10: 3

route-target both 1 0 : 5010

enable

evpn-exit

!

nvi 20

vxlan-id 5020

evpn

rd 20: 3

route-target both 2 0 : 5020

enable

evpn-exit

!

Interface Ethernet s3p11

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet s3p23

switchport access vlan 23

!

Interface Ethernet s3p13

switchport access vlan 13

!

Interface Ethernet s3p34

switchport access vlan 34

!

virtual-equipment-group 1

source ip 34.1.1.3

remote ip 34.1.1.4

!

interface Vlan13

IP address 13.1.1.3 255.255.255.0

!

interface Vlan23

IP address 23.1.1.3 255.255.255.0

!

interface Vlan3 4

IP address 3 4 .1.1.3 255.255.255.0

!

interface Loopback1

IP address 3.3.3.3 255.255.255.255

!

interface Loopback 2

IP address 34.34.34.34 255.255.255.255

!

interface Nvi-interface10

ip vrf forwarding 1

mac-address 00-00-5e-00-00- 01

ipv6 address 2010::254/64

virtual-equipment-group 1

IP address 192.168.10.254 255.255.255.0

!

interface Nvi-interface20

ip vrf forwarding 2

mac-address 00-00-5e-00-00- 02

ipv6 address 20 2 0::254/64

virtual-equipment-group 1

IP address 192.168. 2 0.254 255.255.255.0

!

router ospf 1

passive-interface Loopback1

passive-interface Loopback 2

network 3.3.3.3 0.0.0.0 area 0

network 3 4 .3 4 .3 4 .3 4 0.0.0.0 area 0

network 13.1.1.3 0.0.0.0 area 0

network 23.1.1.3 0.0.0.0 area 0

!

router bgp 100

bgp router-id 3.3.3.3

bgp scan-time 1

neighbor 1.1.1.1 remote-as 100

neighbor 1.1.1.1 update-source 3.3.3.3

neighbor 2.2.2.2 remote-as 100

neighbor 2.2.2.2 update-source 3.3.3.3

address-family ipv4 unicast

neighbor 1.1.1.1 activate

neighbor 2.2.2.2 activate

exit-address-family

address-family l2vpn evpn

neighbor 1.1.1.1 activate

neighbor 2.2.2.2 activate

exit-address-family

address-family ipv4 vrf 1

exit-address-family

address-family ipv6 vrf 1

exit-address-family

address-family ipv4 vrf 2

exit-address-family

address-family ipv6 vrf 2

exit-address-family

!

DUT4

evpn nve source-address 3 4 .3 4 .3 4 .3 4

evpn nve mac-address 00-00-00- 00-00-34

!

vlan 1-4094

!

ip vrf 1

rd 4 3:1

l3-vni 10001

route-target both 1:10001

!

ip vrf 2

rd 4 3:2

l3-vni 10002

route-target both 2:10002

!

nvi 10

vxlan-id 5010

evpn

rd 10: 4

route-target both 1 0 : 5010

enable

evpn-exit

!

nvi 20

vxlan-id 5020

evpn

rd 20: 4

route-target both 2 0 : 5020

enable

evpn-exit

!

Interface Ethernet s4p11

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet s4p24

switchport access vlan 2 4

!

Interface Ethernet s4p14

switchport access vlan 1 4

!

Interface Ethernet s4p34

switchport access vlan 34

!

virtual-equipment-group 1

source ip 34.1.1.4

remote ip 34.1.1.3

!

interface Vlan1 4

ip address 1 4 .1.1. 4 255.255.255.0

!

interface Vlan2 4

ip address 2 4 .1.1. 4 255.255.255.0

!

interface Vlan3 4

IP address 3 4 .1.1. 4 255.255.255.0

!

interface Loopback1

IP address 4.4.4.4 255.255.255.255

!

interface Loopback 2

IP address 34.34.34.34 255.255.255.255

!

interface Nvi-interface10

ip vrf forwarding 1

mac-address 00-00-5e-00-00- 01

ipv6 address 2010::254/64

virtual-equipment-group 1

IP address 192.168.10.254 255.255.255.0

!

interface Nvi-interface20

ip vrf forwarding 2

mac-address 00-00-5e-00-00- 02

ipv6 address 20 2 0::254/64

virtual-equipment-group 1

IP address 192.168. 2 0.254 255.255.255.0

!

router ospf 1

passive-interface Loopback1

passive-interface Loopback 2

network 4.4.4.4 0.0.0.0 area 0

network 3 4 .3 4 .3 4 .3 4 0.0.0.0 area 0

network 14.1.1.4 0.0.0.0 area 0

network 2 4.1.1.4 0.0.0.0 area 0

!

router bgp 100

bgp router-id 4.4.4.4

bgp scan-time 1

neighbor 1.1.1.1 remote-as 100

neighbor 1.1.1.1 update-source 4.4.4.4

neighbor 2.2.2.2 remote-as 100

neighbor 2.2.2.2 update-source 4.4.4.4

address-family ipv4 unicast

neighbor 1.1.1.1 activate

neighbor 2.2.2.2 activate

exit-address-family

address-family l2vpn evpn

neighbor 1.1.1.1 activate

neighbor 2.2.2.2 activate

exit-address-family

address-family ipv4 vrf 1

exit-address-family

address-family ipv6 vrf 1

exit-address-family

address-family ipv4 vrf 2

exit-address-family

address-family ipv6 vrf 2

exit-address-family

!

DUT2

evpn nve source-address 2.2.2.2

evpn nve mac-address 00-00-00- 00-00-22

!

vlan 1-4094

!

vxlan remote mac-address-learning disable

!

nvi 10

vxlan-id 5010

evpn

rd 10: 2

route-target both 1 0 : 5010

enable

evpn-exit

!

nvi 20

vxlan-id 5020

evpn

rd 20: 2

route-target both 2 0 : 5020

enable

evpn-exit

!

Interface Ethernet s2p11

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet s2p24

switchport access vlan 24

!

Interface Ethernet s2p23

switchport access vlan 2 3

!

interface Vlan 2 3

IP address 2 3.1.1. 2 255.255.255.0

!

interface Vlan 24

ip address 24 .1.1. 2 255.255.255.0

!

interface Loopback1

IP address 2.2.2.2 255.255.255.255

!

router ospf 1

passive-interface Loopback1

network 2.2.2.2 0.0.0.0 area 0

network 2 3.1.1. 2 0.0.0.0 area 0

network 24 .1.1. 2 0.0.0.0 area 0

!

router bgp 100

neighbor 3.3.3.3 remote-as 100

neighbor 3.3.3.3 update-source 2.2.2.2

neighbor 4.4.4.4 remote-as 100

neighbor 4.4.4.4 update-source 2.2.2.2

address-family l2vpn evpn

neighbor 3.3.3.3 activate

neighbor 4.4.4.4 activate

exit-address-family

!

S5:

vlan 1-4094

!

Interface s5p15

switchport mode trunk

!

Interface s5p11

switchport mode trunk

ipv4 address vlan10 interface gateway address is 192.168.10.254 ,

vlan20 is 192.168.20.254 ;

the IPv6 address vlan10 interface is 2010::254 .

Vlan20 interface is 2020::254 ;

Tc1

Interface 1 : 192.168.10.10/24 00-00-11-01-00-01 vlan10

Interface 2 : 192.168.20.10/24 00-00-11-02-00-01 vlan20

Interface 3 : 2010::10/64 00-00-11-03-00-01 vlan10

Interface 4 : 2020::10/64 00-00-11-04-00-01 vlan20

Tc 2

Interface 1 : 192.168.10.20/24 00-00-22-01-00-01 vlan10

Interface 2 : 192.168.20.20/24 00-00-22-02-00-01 vlan20

Interface 3 : 2010::20/64 00-00-22-03-00-01 vlan10

Interface 4 : 2020::20/64 00-00-22-04-00-01 vlan20

Tc 3

Interface 1 : 192.168.10.30/24 00-00-33-01-00-01 vlan10

Interface 2 : 192.168.20.30/24 00-00-33-02-00-01 vlan20

Interface 4 : 2010::30/64 00-00-33-03-00-01 vlan10

Interface 5 : 2020::30/64 00-00-33-04-00-01 vlan20

Tc 4

Interface 1 : 192.168.10.40/24 00-00-44-01-00-01 vlan10

Interface 2 : 192.168.20.40/24 00-00-44-02-00-01 vlan20

Interface 3 : 2010::40/64 00-00-44-03-00-01 vlan10

Interface 4 : 2020::40/64 00-00-44-04-00-01 vlan20

Tc 5

Interface 1 : 192.168.10.50/24 00-00-55-01-00-01 vlan10

Interface 2 : 192.168.20.50/24 00-00-55-02-00-01 vlan20

Interface 3 : 2010::50/64 00-00-55-03-00-01 vlan10

Interface 4 : 2020::50/64 00-00-55-04-00-01 vlan20

Dual spine scene Test case

December 11 , 2024​​

20:13

DUT1:

evpn nve source-address 1.1.1.1

evpn nve mac-address 00-00-00- 00-00-11

!

vlan 1-4094

!

ip vrf 1

rd 1 3:1

l3-vni 10001

route-target both 1:10001

!

ip vrf 2

rd 1 3:2

l3-vni 10002

route-target both 2:10002

!

vxlan remote mac-address-learning disable

vxlan remote arp-learning disable

vxlan remote nd-learning disable

!

nvi 10

vxlan-id 5010

evpn

rd 1:1 0

route-target both 1 0 : 5010

enable

evpn-exit

!

nvi 20

vxlan-id 5020

evpn

rd 1 : 20

route-target both 2 0 : 5020

enable

evpn-exit

!

Port-group 1

!

Interface Ethernet s1p11

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet s1p15

port-group 1 mode active

!

Interface Ethernet s1p13

switchport access vlan 13

!

Interface Ethernet s1p14

switchport access vlan 14

!

Interface Port-Channel1

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

interface Vlan13

IP address 13.1.1.1 255.255.255.0

!

interface Vlan1 4

ip address 1 4 .1.1.1 255.255.255.0

!

interface Loopback1

IP address 1.1.1.1 255.255.255.255

!

interface Nvi-interface10

ip vrf forwarding 1

mac-address 00-00-5e-00-00- 01

distributed-gateway enable

ipv6 address 2010::254/64

IP address 192.168.10.254 255.255.255.0

!

interface Nvi-interface20

ip vrf forwarding 1

mac-address 00-00-5e-00-00- 02

distributed-gateway enable

ipv6 address 20 2 0::254/64

IP address 192.168. 2 0.254 255.255.255.0

!

router ospf 1

ospf router-id 1.1.1.1

passive-interface Loopback1

network 1.1.1.1 0.0.0.0 area 0

network 13.1.1.1 0.0.0.0 area 0

network 1 4 .1.1.1 0.0.0.0 area 0

!

router bgp 100

bgp router-id 1.1.1.1

neighbor 3.3.3.3 remote-as 100

neighbor 3.3.3.3 update-source 1.1.1.1

neighbor 4.4.4.4 remote-as 100

neighbor 4.4.4.4 update-source 1.1.1.1

address-family ipv4 unicast

neighbor 3.3.3.3 activate

neighbor 4.4.4.4 activate

exit-address-family

address-family l2vpn evpn

neighbor 3.3.3.3 activate

neighbor 4.4.4.4 activate

exit-address-family

address-family ipv4 vrf 1

exit-address-family

address-family ipv6 vrf 1

exit-address-family

address-family ipv4 vrf 2

exit-address-family

address-family ipv6 vrf 2

exit-address-family

!

DUT2:

evpn nve source-address 2.2.2.2

evpn nve mac-address 00-00-00- 00-00-22

!

vlan 1-4094

!

ip vrf 1

rd 2 3:1

l3-vni 10001

route-target both 1:10001

!

ip vrf 2

rd 2 3:2

l3-vni 10002

route-target both 2:10002

!

vxlan remote mac-address-learning disable

vxlan remote arp-learning disable

vxlan remote nd-learning disable

!

nvi 10

vxlan-id 5010

evpn

rd 2:10

route-target both 1 0 : 5010

enable

evpn-exit

!

nvi 20

vxlan-id 5020

evpn

rd 2:20

route-target both 2 0 : 5020

enable

evpn-exit

!

Interface Ethernet s2p11

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet s2p23

switchport access vlan 23

!

Interface Ethernet s2p24

switchport access vlan 2 4

!

interface Vlan23

IP address 23.1.1.2 255.255.255.0

!

interface Vlan2 4

ip address 2 4 .1.1.2 255.255.255.0

!

interface Loopback1

IP address 2.2.2.2 255.255.255.255

!

interface Nvi-interface10

ip vrf forwarding 1

mac-address 00-00-5e-00-00- 01

distributed-gateway enable

ipv6 address 2010::254/64

IP address 192.168.10.254 255.255.255.0

!

interface Nvi-interface20

ip vrf forwarding 1

mac-address 00-00-5e-00-00- 02

distributed-gateway enable

ipv6 address 20 2 0::254/64

IP address 192.168. 2 0.254 255.255.255.0

!

router ospf 1

ospf router-id 2.2.2.2

passive-interface Loopback1

network 2.2.2.2 0.0.0.0 area 0

network 23.1.1.2 0.0.0.0 area 0

network 2 4 .1.1.2 0.0.0.0 area 0

!

router bgp 100

bgp router-id 2.2.2.2

neighbor 3.3.3.3 remote-as 100

neighbor 3.3.3.3 update-source 2.2.2.2

neighbor 4.4.4.4 remote-as 100

neighbor 4.4.4.4 update-source 2.2.2.2

address-family ipv4 unicast

neighbor 3.3.3.3 activate

neighbor 4.4.4.4 activate

exit-address-family

address-family l2vpn evpn

neighbor 3.3.3.3 activate

neighbor 4.4.4.4 activate

exit-address-family

address-family ipv4 vrf 1

exit-address-family

address-family ipv6 vrf 1

exit-address-family

address-family ipv4 vrf 2

exit-address-family

address-family ipv6 vrf 2

exit-address-family

!

DUT3:

vlan 1-4094

!

Interface Ethernet s3p23

switchport access vlan 23

!

Interface Ethernet s3p13

switchport access vlan 13

!

interface Vlan13

IP address 13.1.1.3 255.255.255.0

!

interface Vlan23

IP address 23.1.1.3 255.255.255.0

!

interface Loopback1

IP address 3.3.3.3 255.255.255.255

!

router ospf 1

passive-interface Loopback1

network 3.3.3.3 0.0.0.0 area 0

network 13.1.1.3 0.0.0.0 area 0

network 23.1.1.3 0.0.0.0 area 0

!

router bgp 100

bgp router-id 3.3.3.3

bgp scan-time 1

neighbor 1.1.1.1 remote-as 100

neighbor 1.1.1.1 update-source 3.3.3.3

neighbor 2.2.2.2 remote-as 100

neighbor 2.2.2.2 update-source 3.3.3.3

address-family ipv4 unicast

neighbor 1.1.1.1 activate

neighbor 2.2.2.2 activate

exit-address-family

address-family l2vpn evpn

neighbor 1.1.1.1 activate

neighbor 1.1.1.1 route-reflector-client

neighbor 2.2.2.2 activate

neighbor 2.2.2.2 route-reflector-client

exit-address-family

!

DUT4:

vlan 1-4094

!

Interface Ethernet s4p14

switchport access vlan 14

!

Interface Ethernet s4p24

switchport access vlan 24

!

interface Vlan1 4

ip address 1 4 .1.1. 4 255.255.255.0

!

interface Vlan2 4

ip address 2 4 .1.1. 4 255.255.255.0

!

interface Loopback1

IP address 4.4.4.4 255.255.255.255

!

router ospf 1

passive-interface Loopback1

network 4.4.4.4 0.0.0.0 area 0

network 1 4 .1.1. 4 0.0.0.0 area 0

network 2 4 .1.1. 4 0.0.0.0 area 0

!

router bgp 100

bgp router-id 4.4.4.4

bgp scan-time 1

neighbor 1.1.1.1 remote-as 100

neighbor 1.1.1.1 update-source 4.4.4.4

neighbor 2.2.2.2 remote-as 100

neighbor 2.2.2.2 update-source 4.4.4.4

address-family ipv4 unicast

neighbor 1.1.1.1 activate

neighbor 2.2.2.2 activate

exit-address-family

address-family l2vpn evpn

neighbor 1.1.1.1 activate

neighbor 1.1.1.1 route-reflector-client

neighbor 2.2.2.2 activate

neighbor 2.2.2.2 route-reflector-client

exit-address-family

!

ipv4 address vlan10 interface gateway address is 192.168.10.254 ,

vlan20 is 192.168.20.254 ;

the IPv6 address vlan10 interface is 2010::254 .

Vlan20 interface is 2020::254 ;

Tc1

Interface 1 : 192.168.10.10/24 00-00-11-01-00-01 vlan10

Interface 2 : 192.168.20.10/24 00-00-11-02-00-01 vlan20

Interface 3 : 2010::10/64 00-00-11-03-00-01 vlan10

Interface 4 : 2020::10/64 00-00-11-04-00-01 vlan20

Tc 2

Interface 1 : 192.168.10.20/24 00-00-22-01-00-01 vlan10

Interface 2 : 192.168.20.20/24 00-00-22-02-00-01 vlan20

Interface 3 : 2010::20/64 00-00-22-03-00-01 vlan10

Interface 4 : 2020::20/64 00-00-22-04-00-01 vlan20

Tc 3

Interface 1 : 192.168.10.30/24 00-00-33-01-00-01 vlan10

Interface 2 : 192.168.20.30/24 00-00-33-02-00-01 vlan20

Interface 4 : 2010::30/64 00-00-33-03-00-01 vlan10

Interface 5 : 2020::30/64 00-00-33-04-00-01 vlan20

Tc 4

Interface 1 : 192.168.10.40/24 00-00-44-01-00-01 vlan10

Interface 2 : 192.168.20.40/24 00-00-44-02-00-01 vlan20

Interface 3 : 2010::40/64 00-00-44-03-00-01 vlan10

Interface 4 : 2020::40/64 00-00-44-04-00-01 vlan20

Tc 5

Interface 1 : 192.168.10.50/24 00-00-55-01-00-01 vlan10

Interface 2 : 192.168.20.50/24 00-00-55-02-00-01 vlan20

Interface 3 : 2010::50/64 00-00-55-03-00-01 vlan10

Interface 4 : 2020::50/64 00-00-55-04-00-01 vlan20

DUT4

evpn nve source-address 3 4 .3 4 .3 4 .3 4

evpn nve mac-address 00-00-00- 00-00-34

!

vlan 1-4094

!

ip vrf 1

rd 4 3:1

l3-vni 10001

route-target both 1:10001

!

ip vrf 2

rd 4 3:2

l3-vni 10002

route-target both 2:10002

!

nvi 10

vxlan-id 5010

evpn

rd 10: 4

route-target both 1 0 : 5010

enable

evpn-exit

!

nvi 20

vxlan-id 5020

evpn

rd 20: 4

route-target both 2 0 : 5020

enable

evpn-exit

!

Interface Ethernet s4p11

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet s4p24

switchport access vlan 2 4

!

Interface Ethernet s4p14

switchport access vlan 1 4

!

Interface Ethernet s4p34

switchport access vlan 34

!

virtual-equipment-group 1

source ip 34.1.1.4

remote ip 34.1.1.3

!

interface Vlan1 4

ip address 1 4 .1.1. 4 255.255.255.0

!

interface Vlan2 4

ip address 2 4 .1.1. 4 255.255.255.0

!

interface Vlan3 4

IP address 3 4 .1.1. 4 255.255.255.0

!

interface Loopback1

IP address 4.4.4.4 255.255.255.255

!

interface Loopback 2

IP address 34.34.34.34 255.255.255.255

!

interface Nvi-interface10

ip vrf forwarding 1

mac-address 00-00-5e-00-00- 01

ipv6 address 2010::254/64

virtual-equipment-group 1

IP address 192.168.10.254 255.255.255.0

!

interface Nvi-interface20

ip vrf forwarding 2

mac-address 00-00-5e-00-00- 02

ipv6 address 20 2 0::254/64

virtual-equipment-group 1

IP address 192.168. 2 0.254 255.255.255.0

!

router ospf 1

passive-interface Loopback1

passive-interface Loopback 2

network 3.3.3.3 0.0.0.0 area 0

network 3 4 .3 4 .3 4 .3 4 0.0.0.0 area 0

network 14.1.1.4 0.0.0.0 area 0

network 2 4.1.1.4 0.0.0.0 area 0

!

router bgp 100

neighbor 1.1.1.1 remote-as 100

neighbor 1.1.1.1 update-source 4.4.4.4

neighbor 2.2.2.2 remote-as 100

neighbor 2.2.2.2 update-source 4.4.4.4

address-family l2vpn evpn

neighbor 1.1.1.1 activate

neighbor 2.2.2.2 activate

exit-address-family

address-family ipv4 vrf 1

exit-address-family

address-family ipv4 vrf 2

exit-address-family

!

DUT2

evpn nve source-address 2.2.2.2

evpn nve mac-address 00-00-00- 00-00-22

!

vlan 1-4094

!

nvi 10

vxlan-id 5010

evpn

rd 10: 2

route-target both 1 0 : 5010

enable

evpn-exit

!

nvi 20

vxlan-id 5020

evpn

rd 20: 2

route-target both 2 0 : 5020

enable

evpn-exit

!

Interface Ethernet s2p11

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet s2p24

switchport access vlan 24

!

Interface Ethernet s2p23

switchport access vlan 2 3

!

interface Vlan 2 3

IP address 2 3.1.1. 2 255.255.255.0

!

interface Vlan 24

ip address 24 .1.1. 2 255.255.255.0

!

interface Loopback1

IP address 2.2.2.2 255.255.255.255

!

router ospf 1

passive-interface Loopback1

network 2.2.2.2 0.0.0.0 area 0

network 2 3.1.1. 2 0.0.0.0 area 0

network 24 .1.1. 2 0.0.0.0 area 0

!

router bgp 100

neighbor 3.3.3.3 remote-as 100

neighbor 3.3.3.3 update-source 2.2.2.2

neighbor 4.4.4.4 remote-as 100

neighbor 4.4.4.4 update-source 2.2.2.2

address-family l2vpn evpn

neighbor 3.3.3.3 activate

neighbor 4.4.4.4 activate

exit-address-family

!

S5:

vlan 1-4094

!

Interface s5p15

switchport mode trunk

!

Interface s5p11

switchport mode trunk

Distributed Gateway Dual-Homed Access (Mclag) Test Case

April 3 , 2024​​

9:34

DUT1

!

Interface Ethernet0

ip address 172.18 .1 00 .1 255.255.255.0

!

evpn nve source-address 12.12.12.12

evpn nve mac-address 00-00-00- 00-00-12

!

vlan 1-4094

!

vxlan remote mac-address-learning disable

vxlan remote arp-learning disable

vxlan remote nd-learning disable

!

ip vrf 1

rd 31 :1

l3-vni 10001

route-target both 1:10001

!

ip vrf 2

rd 31 :2

l3-vni 10002

route-target both 2:10002

!

nvi 10

vxlan-id 5010

evpn

rd 10: 1

route-target both 1 0 : 5010

enable

evpn-exit

!

nvi 20

vxlan-id 5020

evpn

rd 20: 1

route-target both 2 0 : 5020

enable

evpn-exit

!

port-group 1

!

port-group 1 28

!

Interface Ethernet s1p11

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet s1p15

port-group 1 mode active

!

Interface Ethernet s1p1-4

switchport mode trunk

port-group 128 mode on

!

Interface Ethernet s1p13

switchport access vlan 13

!

virtual-equipment-group 1

!

mclag

mclag enable

mclag domain-id 1

mclag local-ip 10.10.1.1

mclag peer-ip 10.10.1.2

mclag priority 64

virtual-equipment-group 1

!

Interface Port-Channel 1

mclag group

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Port-Channel128

switchport mclag data link

!

interface Vlan13

IP address 13.1.1. 1 255.255.255.0

!

interface Loopback1

IP address 1.1.1.1 255.255.255.255

!

interface Loopback 2

IP address 12.12.12.12 255.255.255.255

!

interface Vlan4094

IP address 10.10.1.1 255.255.255.252

!

interface Nvi-interface10

ip vrf forwarding 1

mac-address 00-00-5e-00-00- 01

distributed-gateway enable

ipv6 address 2010::254/64

virtual-equipment-group 1

IP address 192.168.10.254 255.255.255.0

!

interface Nvi-interface20

ip vrf forwarding 2

mac-address 00-00-5e-00-00- 02

distributed-gateway enable

ipv6 address 20 2 0::254/64

virtual-equipment-group 1

IP address 192.168. 2 0.254 255.255.255.0

!

router ospf 1

passive-interface Loopback1

passive-interface Loopback 2

network 1.1.1.1 0.0.0.0 area 0

network 12.12.12.12 0.0.0.0 area 0

network 13.1.1. 1 0.0.0.0 area 0

!

router bgp 100

bgp router-id 1.1.1.1

neighbor 3.3.3.3 remote-as 100

neighbor 3.3.3.3 update-source 1.1.1.1

address-family ipv4 unicast

neighbor 3.3.3.3 activate

exit-address-family

address-family l2vpn evpn

neighbor 3.3.3.3 activate

exit-address-family

address-family ipv4 vrf 1

exit-address-family

address-family ipv6 vrf 1

exit-address-family

address-family ipv4 vrf 2

exit-address-family

address-family ipv6 vrf 2

exit-address-family

!

DUT2

!

Interface Ethernet0

ip address 172.18 .1 00 . 2 255.255.255.0

!

evpn nve source-address 12.12.12.12

evpn nve mac-address 00-00-00- 00-00-12

!

vlan 1-4094

!

vxlan remote mac-address-learning disable

vxlan remote arp-learning disable

vxlan remote nd-learning disable

!

ip vrf 1

rd 32 :1

l3-vni 10001

route-target both 1:10001

!

ip vrf 2

rd 32 :2

l3-vni 10002

route-target both 2:10002

!

nvi 10

vxlan-id 5010

evpn

rd 10: 2

route-target both 1 0 : 5010

enable

evpn-exit

!

nvi 20

vxlan-id 5020

evpn

rd 20: 2

route-target both 2 0 : 5020

enable

evpn-exit

!

port-group 1

!

port-group 1 28

!

Interface Ethernet s2p11

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet s2p25

port-group 1 mode active

!

Interface Ethernet s1p1-4

switchport mode trunk

port-group 128 mode on

!

Interface Ethernet s2p23

switchport access vlan 2 3

!

virtual-equipment-group 1

!

mclag

mclag enable

mclag domain-id 1

mclag local-ip 10.10.1.2

mclag peer-ip 10.10.1.1

virtual-equipment-group 1

!

Interface Port-Channel 1

mclag group

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Port-Channel128

switchport mclag data link

!

interface Vlan 2 3

IP address 2 3.1.1. 2 255.255.255.0

!

interface Loopback1

IP address 2.2.2.2 255.255.255.255

!

interface Loopback 2

IP address 12.12.12.12 255.255.255.255

!

interface Vlan4094

IP address 10.10.1. 2 255.255.255.252

!

interface Nvi-interface10

ip vrf forwarding 1

mac-address 00-00-5e-00-00- 01

distributed-gateway enable

ipv6 address 2010::254/64

virtual-equipment-group 1

IP address 192.168.10.254 255.255.255.0

!

interface Nvi-interface20

ip vrf forwarding 2

mac-address 00-00-5e-00-00- 02

distributed-gateway enable

ipv6 address 20 2 0::254/64

virtual-equipment-group 1

IP address 192.168. 2 0.254 255.255.255.0

!

router ospf 1

passive-interface Loopback1

passive-interface Loopback 2

network 2.2.2.2 0.0.0.0 area 0

network 12.12.12.12 0.0.0.0 area 0

network 2 3.1.1. 2 0.0.0.0 area 0

!

router bgp 100

bgp router-id 2.2.2.2

neighbor 3.3.3.3 remote-as 100

neighbor 3.3.3.3 update-source 2.2.2.2

address-family ipv4 unicast

neighbor 3.3.3.3 activate

exit-address-family

address-family l2vpn evpn

neighbor 3.3.3.3 activate

exit-address-family

address-family ipv4 vrf 1

exit-address-family

address-family ipv6 vrf 1

exit-address-family

address-family ipv4 vrf 2

exit-address-family

address-family ipv6 vrf 2

exit-address-family

!

DUT3

!

evpn nve source-address 3.3.3.3

evpn nve mac-address 00-00-00- 00-00-33

!

vlan 1-4094

!

vxlan remote mac-address-learning disable

vxlan remote arp-learning disable

vxlan remote nd-learning disable

!

ip vrf 1

rd 3 3:1

l3-vni 10001

route-target both 1:10001

!

ip vrf 2

rd 3 3:2

l3-vni 10002

route-target both 2:10002

!

nvi 10

vxlan-id 5010

evpn

rd 10: 3

route-target both 1 0 : 5010

enable

evpn-exit

!

nvi 20

vxlan-id 5020

evpn

rd 20: 3

route-target both 2 0 : 5020

enable

evpn-exit

!

Interface Ethernet s3p11

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet s3p23

switchport access vlan 23

!

Interface Ethernet s3p13

switchport access vlan 13

!

interface Vlan13

IP address 13.1.1.3 255.255.255.0

!

interface Vlan23

IP address 23.1.1.3 255.255.255.0

!

interface Loopback1

IP address 3.3.3.3 255.255.255.255

!

interface Nvi-interface10

ip vrf forwarding 1

mac-address 00-00-5e-00-00- 01

distributed-gateway enable

ipv6 address 2010::254/64

IP address 192.168.10.254 255.255.255.0

!

interface Nvi-interface20

ip vrf forwarding 2

mac-address 00-00-5e-00-00- 02

distributed-gateway enable

ipv6 address 20 2 0::254/64

IP address 192.168. 2 0.254 255.255.255.0

!

router ospf 1

passive-interface Loopback1

network 3.3.3.3 0.0.0.0 area 0

network 13.1.1.3 0.0.0.0 area 0

network 23.1.1.3 0.0.0.0 area 0

!

!

router bgp 100

bgp router-id 3.3.3.3

neighbor 1.1.1.1 remote-as 100

neighbor 1.1.1.1 update-source 3.3.3.3

neighbor 2.2.2.2 remote-as 100

neighbor 2.2.2.2 update-source 3.3.3.3

address-family ipv4 unicast

neighbor 1.1.1.1 activate

neighbor 2.2.2.2 activate

exit-address-family

address-family l2vpn evpn

neighbor 1.1.1.1 activate

neighbor 1.1.1.1 route-reflector-client

neighbor 2.2.2.2 activate

neighbor 2.2.2.2 route-reflector-client

exit-address-family

address-family ipv4 vrf 1

exit-address-family

address-family ipv6 vrf 1

exit-address-family

address-family ipv4 vrf 2

exit-address-family

address-family ipv6 vrf 2

exit-address-family

!

S5:

vlan 1-4094

!

port-group 1

!

Interface s5p15

switchport mode trunk

port-group 1 mode active

!

Interface s5p25

switchport mode trunk

port-group 1 mode active

!

Interface s5p11

switchport mode trunk

ipv4 address vlan10 interface gateway address is 192.168.10.254 ,

vlan20 is 192.168.20.254 ;

the IPv6 address vlan10 interface is 2010::254 .

Vlan20 interface is 2020::254 ;

Tc1

Interface 1 : 192.168.10.10/24 00-00-11-01-00-01 vlan10

Interface 2 : 192.168.20.10/24 00-00-11-02-00-01 vlan20

Interface 3 : 2010::10/64 00-00-11-03-00-02 vlan10

Interface 4 : 2020::10/64 00-00-11-04-00-02 vlan20

Tc2

Interface 1 : 192.168.10.20/24 00-00-22-01-00-01 vlan10

Interface 2 : 192.168.20.20/24 00-00-22-02-00-01 vlan20

Interface 3 : 2010::20/64 00-00-22-03-00-02 vlan10

Interface 4 : 2020::20/64 00-00-22-04-00-02 vlan20

Tc3

Interface 1 : 192.168.10.30/24 00-00-33-01-00-01 vlan10

Interface 2 : 192.168.20.30/24 00-00-33-02-00-01 vlan20

Interface 3 : 2010::30/64 00-00-33-03-00-02 vlan10

Interface 4 : 2020::30/64 00-00-33-04-00-02 vlan20

Tc5

Interface 1 : 192.168.10.50/24 00-00-55-01-00-01 vlan10

Interface 2 : 192.168.20.50/24 00-00-55-02-00-01 vlan20

Interface 3 : 2010::50/64 00-00-55-03-00-02 vlan10

Interface 4 : 2020::50/64 00-00-55-04-00-02 vlan20

ESI access scenario Test case

April 3 , 2024​​

9:34

DUT1

evpn nve source-address 1.1.1.1

evpn nve mac-address 00-00-00- 00-00-11

!

vlan 1-4094

!

vxlan remote mac-address-learning disable

vxlan remote arp-learning disable

vxlan remote nd-learning disable

!

ip vrf 1

rd 31 :1

l3-vni 10001

route-target both 1:10001

!

ip vrf 2

rd 31 :2

l3-vni 10002

route-target both 2:10002

!

nvi 10

vxlan-id 5010

evpn

rd 1:10

route-target both 1 0 : 5010

enable

evpn-exit

!

nvi 20

vxlan-id 502 1

evpn

rd 1:20

route-target both 2 0 : 5021

enable

evpn-exit

!

port-group 1

!

Interface Ethernet s1p11

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet s1p15

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

esi 0000.0001.0002.0003.0004

!

Interface Ethernet s1p13

switchport access vlan 13

!

interface Vlan13

IP address 13.1.1. 1 255.255.255.0

!

interface Loopback1

IP address 1.1.1.1 255.255.255.255

!

interface Loopback 2

IP address 12.12.12.12 255.255.255.255

!

interface Nvi-interface10

ip vrf forwarding 1

mac-address 00-00-5e-00-00- 01

distributed-gateway enable

ipv6 address 2010::254/64

IP address 192.168.10.254 255.255.255.0

!

interface Nvi-interface20

ip vrf forwarding 1

mac-address 00-00-5e-00-00- 02

distributed-gateway enable

ipv6 address 20 2 0::254/64

IP address 192.168. 2 0.254 255.255.255.0

!

router ospf 1

passive-interface Loopback1

passive-interface Loopback 2

network 1.1.1.1 0.0.0.0 area 0

network 12.12.12.12 0.0.0.0 area 0

network 13.1.1. 1 0.0.0.0 area 0

!

router bgp 100

bgp router-id 1.1.1.1

neighbor 3.3.3.3 remote-as 100

neighbor 3.3.3.3 update-source 1.1.1.1

address-family ipv4 unicast

neighbor 3.3.3.3 activate

exit-address-family

address-family l2vpn evpn

neighbor 3.3.3.3 activate

exit-address-family

address-family ipv4 vrf 1

exit-address-family

address-family ipv6 vrf 1

exit-address-family

address-family ipv4 vrf 2

exit-address-family

address-family ipv6 vrf 2

exit-address-family

!

DUT2

evpn nve source-address 2.2.2.2

evpn nve mac-address 00-00-00- 00-00-22

!

vlan 1-4094

!

vxlan remote mac-address-learning disable

vxlan remote arp-learning disable

vxlan remote nd-learning disable

!

ip vrf 1

rd 32 :1

l3-vni 10001

route-target both 1:10001

!

ip vrf 2

rd 32 :2

l3-vni 10002

route-target both 2:10002

!

nvi 10

vxlan-id 5010

evpn

rd 2:10

route-target both 1 0 : 5010

enable

evpn-exit

!

nvi 20

vxlan-id 502 1

evpn

rd 2:20

route-target both 2 0 : 5021

enable

evpn-exit

!

port-group 1

!

Interface Ethernet s2p11

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet s2p25

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

esi 0000.0001.0002.0003.0004

!

Interface Ethernet s2p23

switchport access vlan 2 3

!

interface Vlan 2 3

IP address 2 3.1.1. 2 255.255.255.0

!

interface Loopback1

IP address 2.2.2.2 255.255.255.255

!

interface Loopback 2

IP address 12.12.12.12 255.255.255.255

!

interface Nvi-interface10

ip vrf forwarding 1

mac-address 00-00-5e-00-00- 01

distributed-gateway enable

ipv6 address 2010::254/64

IP address 192.168.10.254 255.255.255.0

!

interface Nvi-interface20

ip vrf forwarding 1

mac-address 00-00-5e-00-00- 02

distributed-gateway enable

ipv6 address 20 2 0::254/64

IP address 192.168. 2 0.254 255.255.255.0

!

router ospf 1

passive-interface Loopback1

passive-interface Loopback 2

network 2.2.2.2 0.0.0.0 area 0

network 12.12.12.12 0.0.0.0 area 0

network 2 3.1.1. 2 0.0.0.0 area 0

!

router bgp 100

bgp router-id 2.2.2.2

neighbor 3.3.3.3 remote-as 100

neighbor 3.3.3.3 update-source 2.2.2.2

address-family ipv4 unicast

neighbor 3.3.3.3 activate

exit-address-family

address-family l2vpn evpn

neighbor 3.3.3.3 activate

exit-address-family

address-family ipv4 vrf 1

exit-address-family

address-family ipv6 vrf 1

exit-address-family

address-family ipv4 vrf 2

exit-address-family

address-family ipv6 vrf 2

exit-address-family

!

DUT3

evpn nve source-address 3.3.3.3

evpn nve mac-address 00-00-00- 00-00-33

!

vlan 1-4094

!

vxlan remote mac-address-learning disable

vxlan remote arp-learning disable

vxlan remote nd-learning disable

!

ip vrf 1

rd 3 3:1

l3-vni 10001

route-target both 1:10001

!

ip vrf 2

rd 3 3:2

l3-vni 10002

route-target both 2:10002

!

nvi 10

vxlan-id 5010

evpn

rd 3:10

route-target both 1 0 : 5010

enable

evpn-exit

!

nvi 20

vxlan-id 502 1

evpn

rd 3:20

route-target both 2 0 : 5021

enable

evpn-exit

!

Interface Ethernet s3p11

switchport mode trunk

xconnect nvi 10 mode vlan svid 10

xconnect nvi 20 mode vlan svid 20

!

Interface Ethernet s3p23

switchport access vlan 23

!

Interface Ethernet s3p13

switchport access vlan 13

!

interface Vlan13

IP address 13.1.1.3 255.255.255.0

!

interface Vlan23

IP address 23.1.1.3 255.255.255.0

!

interface Loopback1

IP address 3.3.3.3 255.255.255.255

!

interface Nvi-interface10

ip vrf forwarding 1

mac-address 00-00-5e-00-00- 01

distributed-gateway enable

ipv6 address 2010::254/64

IP address 192.168.10.254 255.255.255.0

!

interface Nvi-interface20

ip vrf forwarding 1

mac-address 00-00-5e-00-00- 02

distributed-gateway enable

ipv6 address 20 2 0::254/64

IP address 192.168. 2 0.254 255.255.255.0

!

router ospf 1

passive-interface Loopback1

network 3.3.3.3 0.0.0.0 area 0

network 13.1.1.3 0.0.0.0 area 0

network 23.1.1.3 0.0.0.0 area 0

!

router bgp 100

bgp router-id 3.3.3.3

neighbor 1.1.1.1 remote-as 100

neighbor 1.1.1.1 update-source 3.3.3.3

neighbor 2.2.2.2 remote-as 100

neighbor 2.2.2.2 update-source 3.3.3.3

address-family ipv4 unicast

neighbor 1.1.1.1 activate

neighbor 2.2.2.2 activate

exit-address-family

address-family l2vpn evpn

neighbor 1.1.1.1 activate

neighbor 1.1.1.1 route-reflector-client

neighbor 2.2.2.2 activate

neighbor 2.2.2.2 route-reflector-client

exit-address-family

address-family ipv4 vrf 1

exit-address-family

address-family ipv6 vrf 1

exit-address-family

address-family ipv4 vrf 2

exit-address-family

address-family ipv6 vrf 2

exit-address-family

!

S5:

vlan 1-4094

!

port-group 1

!

Interface s5p15

switchport mode trunk

port-group 1 mode on

!

Interface s5p25

switchport mode trunk

port-group 1 mode on

!

Interface s5p11

switchport mode trunk

ipv4 address vlan10 interface gateway address is 192.168.10.254 ,

vlan20 is 192.168.20.254 ;

the IPv6 address vlan10 interface is 2010::254 .

Vlan20 interface is 2020::254 ;

Tc1

Interface 1 : 192.168.10.10/24 00-00-11-01-00-01 vlan10

Interface 2 : 192.168.20.10/24 00-00-11-02-00-01 vlan20

Interface 3 : 2010::10/64 00-00-11-03-00-02 vlan10

Interface 4 : 2020::10/64 00-00-11-04-00-02 vlan20

Tc2

Interface 1 : 192.168.10.20/24 00-00-22-01-00-01 vlan10

Interface 2 : 192.168.20.20/24 00-00-22-02-00-01 vlan20

Interface 3 : 2010::20/64 00-00-22-03-00-02 vlan10

Interface 4 : 2020::20/64 00-00-22-04-00-02 vlan20

Tc3

Interface 1 : 192.168.10.30/24 00-00-33-01-00-01 vlan10

Interface 2 : 192.168.20.30/24 00-00-33-02-00-01 vlan20

Interface 3 : 2010::30/64 00-00-33-03-00-02 vlan10

Interface 4 : 2020::30/64 00-00-33-04-00-02 vlan20

Tc5

Interface 1 : 192.168.10.50/24 00-00-55-01-00-01 vlan10

Interface 2 : 192.168.20.50/24 00-00-55-02-00-01 vlan20

Interface 3 : 2010::50/64 00-00-55-03-00-02 vlan10

Interface 4 : 2020::50/64 00-00-55-04-00-02 vlan20